MATERIAL  SAFETY  DATA  SHEET

(form according to EEC Directive 93/112/EC)

NAME :  LITHIUM, THIONYL CHLORIDE (Li-SOCl2)

NON-RECHARGEABLE BATTERIES

1 - IDENTIFICATION  (of the product and the supplier)

1.1.  Product :			             Rechargeable battery

| yes |  |
| --- | --- |
| no | X |

Trade name and model  : 	   LITHIUM, THIONYL CHLORIDE (Li-SOCl2)

NON-RECHARGEABLE BATTERIES

IEC designation  :

Models :   LS and LSH series	LS 14250,  LS 14250 C,  LS 14500,  LS 14500 C,

LS 17500,  LS 26500,  LS 26500 C,  LS 33600,

LS 33600 C,  LSH 26180,  LSH 14,  LSH 20

Electrochemical system  :

| Electrodes | Negative electrode Lithium metal (Li) | Positive electrode Thionyl chloride (SOCl2) |
| --- | --- | --- |
| Electrolyte | Solution of lithium (LiAlCl4) in | tetrachloroaluminate* thionyl chloride |
| Nominal voltage | 3.6 | Volt |

* Lithium tetrachloroaluminate is a combination of lithium chloride (LiCl) and aluminum

chloride (AlCl3).

1.2 - Supplier :

Name  :			SAFT

Address  :			Rue Georges Leclanché - BP 1039    86060

Poitiers Cedex 09 - France

Phone :  33 (0)5 49 55 48 48

Fax :      33 (0)5 49 55 48 50

1.3. - Emergency contact  :		Poitiers Plant Manager  33 (0)5 49 55 51 01

MSDS  2/8

2 - COMPOSITION             (typical weight percentages of basic material

within undischarged individual cells)

| Metals | % | Plastics | % | Others | % |
| --- | --- | --- | --- | --- | --- |
| - Steel (Iron 30 % Nickel 10 %) - Lithium | 35-40* 4-5* | - PVC | 2-3* | - Thionyl chloride - Carbon - Glass - Aluminum chloride - Salts | 40-46* 4 1-4* 1.5* 4-7* |

* % slightly depending from cell type.

3 - HAZARDS IDENTIFICATION

3.1 - Physical :

The Lithium-Thionyl chloride batteries described in this Material Safety Data Sheet are sealed units which are not hazardous when used according to the recommendations of the manufacturer.

Under normal conditions of use, the electrode materials and liquid electrolyte they contain are non-reactive provided the battery integrity is maintained and seals remain intact. Risk of exposure only in case of abuse (mechanical, thermal, electrical) which leads to the activation of safety valves and/or the rupture of the battery containers. Electrolyte leakage, electrode materials reaction with moisture/water or battery vent/explosion/fire may follow, depending upon the circumstances.

3.2 - Chemical :

Classification of dangerous substances contained into the product

as per directive   67/548/EEC

| Substance |  | Content* | Melting point |  | Classi | fication |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N° EEC (CAS N°) | Chemical symbol | Mass % |  | Letter | Indication of danger | Special risk (1) | Safety advice (2) |
| 231 102 5 (7439-93-2) | Li | 4-5 | 180.5°C | F | Corrosive Flammable | R14/15 R21 R22 R41 R43 | S2 S8 S22 S24 S26 S37 S43 S45 |
| 231 748 8 (7719-09-7) | SOCl2 | 40-46 | -104.5°C | C | Irritant Corrosive Harmful | R14 R21 R22 R35 R37 R41 R42/43 | S2 S8 S24 S26 S36 S37 S45 |
| (00744-67-00) | AlCl3 |  1.5 | 190°C | C | Irritant Corrosive | R14 R22 R37 R41 R43 | S2 S8 S22 S24 S26 S36 S45 |

* slight variations depending from cell type.

MSDS  3/8

1 - Nature of special risks :

R 14/15 	Reacts with water and yields flammable gases

R 21		Harmful in contact with skin

R 22		Harmful is swallowed

R 35		Causes severe burns

R 41		Risk of serious damage to the eye

R 42/43		May cause sensitization by inhalation and skin contact

R 43		May cause sensitization by skin contact

2 - Safety advices :

S 2 		Keep out of reach from children

S 8		Keep away from moisture

S 22		Do not breathe dust

S 24		Avoid contact with skin

S 26		In case of contact with eyes, rinse immediately with plenty of water

and seek medical attention

S 36		Wear suitable protective clothing

S 37		Wear suitable gloves

S 45		In case of incident, seek medical attention.

4 - FIRST AID MEASURES

In case of battery rupture or explosion, evacuate personnel from contaminated area and provide maximum ventilation to clear out corrosive fumes/gases and pungent odour.

In all case, seek immediate medical attention.

Eye contact :      	Flush with plenty of water (eyelids held open) for at least 15 minutes.

Skin contact :    	Remove all contaminated clothing and flush affected areas with plenty

of water and soap for at least 15 minutes.

Do not apply greases or ointments.

Ingestion : 		Dilute by giving plenty of water and get immediate medical attention.

Assure that the victim does not aspirate vomited material by use of

positional drainage.

Assure that mucus does not obstruct the airway.

Do not give anything by mouth to an unconscious person.

Inhalation :       	Remove to fresh air and ventilate the contaminated area.

Give oxygen or artificial respiration if needed.

MSDS  4/8

5 - FIRE-FIGHTING MEASURES

Fire and explosion hazard :  The battery can spout vaporized or decomposed

electrolyte fumes in case of exposure above 100°C

resulting from unappropriate use or the environment.

Risk of explosion is increased if the melting point of

lithium (180.5°C) is exceeded.

Hydrogen coming from the decomposition of lithium

metal with water is flammable.

Extinguishing media :  	  Suitable :  	     Type D extinguishers,  Lith-X

Water may be used only to keep battery cool.

Not to be used :  Water in case of battery rupture or

explosion (detectable by the pungent odour).

Special exposure hazards :    Following cell overheating due to external source or

due to unproper use, electrolyte leakage or battery

container rupture may occur and release inner

component/material in the environment.

Eye contact :  The electrolyte solution contained in the

battery is corrosive to all ocular tissues.

Skin contact :  The electrolyte solution contained in the

battery is corrosive and causes skin irritation and burns.

Ingestion :  The ingestion of electrolyte solution causes

tissue damage to throat and gastro/respiratory tract.

Inhalation :  Contents of a leaking or ruptured battery

can cause respiratory tract, mucus, membrane irritation

and edema.

Special protective  		  Use self-contained breathing apparatus to avoid

equipment  :		  breathing irritant fumes.

Wear protective clothing and equipment to prevent

body contact with electrolyte solution.

6 - ACCIDENTAL RELEASE MEASURES

The material contained within the batteries would only be expelled under abusive conditions.

Using shovel or broom, cover battery or spilled substances with dry sand or, preferably, sodium carbonate (Na2CO3) or 1:1 mixture of soda ash and slaked slime. Keep away from water, rain, snow. Place in approved container (after cooling if necessary) and dispose in accordance with local regulations.

MSDS  5/8

7 - HANDLING AND STORAGE

The batteries should not be opened, destroyed nor incinerated since they may leak or rupture and release in the environment the ingredients they contain (see Section 6).

Handling :  Do not crush, pierce, short (+) and (-) battery terminals with conductive (i.e. metal) goods. Do not directly heat or solder. Do not throw into fire. Do not mix batteries of different types and brands. Do not mix new and used batteries. Keep batteries in non conductive (i.e. plastic) trays.

Storage :  Store in a cool (preferably below 30°C) and ventilated area away from moisture, sources of heat, open flames, food and drink. Keep adequate clearance between walls and batteries. Temperature above 100°C may result in battery leakage and rupture. Since short circuit can cause burn, leakage and rupture hazard, keep batteries in original packaging until use and do not jumble them.

Other :   Lithium-Thionyl chloride batteries are not rechargeable and should not be tentatively charged.

Follow Manufacturers recommendations regarding maximum recommended currents and operating temperature range.

Applying pressure on deforming the battery may lead to disassembly followed by eye, skin and throat irritation.

8 - EXPOSURE CONTROLS/PERSONAL PROTECTION

Respiratory protection :  	Not necessary under normal use.  In case of battery

rupture, use self contained full-face respiratory

equipment with type ABEK filter.

Hand protection :        	Not necessary under normal use. Use Viton rubber

gloves if handling a leaking or ruptured battery.

Eye protection :               	Not necessary under normal use. Wear safety goggles or

glasses with side shields if handling a leaking or ruptured

battery.

Skin protection :   		Not necessary under normal use Use rubber apron and

protective working in case of handling of a ruptured

battery.

9 - PHYSICAL AND CHEMICAL PROPERTIES

9.1 Appearance :		(Physical shape and color as supplied)

Small metal cylinders, hermetically sealed and fitted

with an external plastic sleeving.

MSDS  6/8

9.2 Temperature range :

|  | Continuous | Occasional |
| --- | --- | --- |
| in storage during discharge during charge | + 30°C max -60/+85°C NA | -60/+100°C -60/+100°C NA |

9.3 Specific energy : 		400- 700 Wh/kg,  depending on cell type

(Note :  Wh = Nominal voltage x rated Ah as defined in

IEC  Standard N° 285.   Kg = Average battery weight)

9.4 Specific instant power : 	2-10 Wh/kg,  depending on cell type

(Note :  0.5xnominal voltagexIp  with Ip = current in

Amperes delivered by a non discharged battery to half

the nominal voltage in 1 second)

9.5 Mechanical resistance : 	As defined in relevant IEC Standard

9.6 Other :

10 - STABILITY AND REACTIVITY

Conditions to avoid :  	Heat above 100°C or incinerate.

Deform, mutilate, crush, pierce, disassemble, recharge.

Short circuit.

Prolonged exposure to humid conditions.

Materials to avoid : 		Oxidizing agents, alkalis, water.

Avoid electrolyte contact with aluminum or zinc.

Hazardous decomposition products :

Hydrogen (H2) as well as lithium oxide (Li2O) and lithium hydroxide (LiOH) dust is produced in case of reaction of lithium metal with water..

Chlorine (Cl2), sulfur dioxide (SO2) and disulfur dichloride (S2Cl2) are produced in case of thermal decomposition of thionyl chloride above 140°C.

Hydrochloric acid (HCl) and sulfur dioxide (SO2) are produced in case of reaction of thionyl chloride with water at room temperature.

Hydrochloric acid (HCl) fumes, lithium oxide, (Li2O), lithium hydroxide (LiOH) and aluminum hydroxide (Al(OH)3) dust are produced in case of reaction of lithium thetrachloroaluminate with water.

MSDS  7/8

11 - TOXOLOGICAL INFORMATION

The Saft primary Lithium-Thionyl chloride batteries do not contain toxic materials.

12 - ECOLOGICAL INFORMATION

When properly used or disposed, the Lithium-Thionyl chloride batteries do not present environmental hazard.

13 - DISPOSAL CONSIDERATIONS

Dispose in accordance with applicable regulations which vary from country to country.

(In most countries, the thrashing of used batteries is forbidden and the end-users are invited to dispose them properly, eventually through non profit organizations, mandated by local governments or organized on a voluntary basis by professionals).

Lithium batteries should have their terminals insulated prior to disposal.

13.1 . Incineration :		Incineration should never be performed by battery users

but eventually by trained professionals in authorized

facilities with proper gas and fumes treatment.

13.2 . Landfilling :		Leachability regulations  (mg/l)

| Component | Leachability | EC limit | EPA | Other* |
| --- | --- | --- | --- | --- |
| Iron Nickel | 100 500 | 2 |  | 5 0.5 |

* applicable to France

13.3 . Recycling :		Send to authorized recycling facilities, eventually through

licensed waste carrier.

14 - TRANSPORT INFORMATION

14.1 United Nations :	UN N°			3090

Classification 		9

Packaging		ICAO 903  for Air Transport

IMDG        for Sea Transport

MSDS  8/8

14.1 International conventions :

Air  			IATA			Yes

Sea			IMDG			Yes

Land			ADR  (road)		Yes

RID    (rail)		Yes

14.3 Other :			in the USA  		Code of Federal Regulations

(49 CFR Ch. 1  § 173-185)

15 - REGULATORY INFORMATION

The transport of lithium batteries is regulated by the United Nations as detailed in the "Model Regulations on the Transport of Dangerous Goods  Ref. ST/SG/AC.10/1  Revision 11  1999".

Depending on their lithium metal content, design, and ability to pass safety tests defined by UN in the "Recommendations on the Transport of Dangerous Goods  Chapter 38.3 Manual of Tests and Criteria  Ref. ST/SG/AC. 10/11  Third Revised Edition 1999",  the Lithium-thionyl chloride cells and the battery packs may or may not be assigned to the UN N° 3090 Class-9 that is restricted for transport.

Individual Lithium-thionyl chloride cells with less than 0.5 gram of lithium metal content are not restricted for transport.

16 - OTHER INFORMATION / DISCLAIMER

This information has been compiled from sources considered to be dependable and is, to the best of our knowledge and belief, accurate and reliable as of the date compiled. However, no representation, warranty (either expressed or implied) or guarantee is made to the accuracy, reliability or completeness of the information contained herein.

This information relates to the specific materials designated and may not be valid for such material used in combination with any other materials or in any process. It is the user's responsibility to satisfy himself as to the suitability and completeness of this information for his particular use.

Saft does not accept liability for any loss or damage that may occur, whether direct, indirect, incidental or consequential, from the use of this information. Saft does not offer warranty against patent infringement. Additional information is available by calling the telephone number above designated purpose.

Revision 2       Date  06/2000

(Previous issue  Revision 1   06/1994)		Signed  ___________________________

Lithium Product Manager